Technical Information

The theatre has a warm, intimate and friendly atmosphere with a gently raking auditorium seating 558.

Lighting and sound are operated from an open control gallery at the rear of the auditorium.

The Stage

A proscenium arch stage with a 3deg rake and mdf covering painted dark grey.

Working depth - 5.45m from cyclorama to proscenium arch plus 3.45m

apron making a total working depth of 8.9m

Working width - 8m

Max working height - 4.5m clear

Please note we are a hemp house and have no counterweight flying system.

Maximum cloth drop is 5.5m, minimum cloth width 9.5m

Load In

Load in is straightforward via stage level double dock doors accessed from the crew car park off Tower Road ( trooper.diverged.insist ) We have a drop down loading platform and a 2.5m long Ralloy steel ramp.

FOH MIX POSITION

A foh mix position is available to visiting lighting and sound technicians at the rear of the auditorium. This is a flat table area constructed across the rows of seats measuring 1.2m x 2.4m and is situated immediately adjacent to the left of the centre aisle.

Sound

We have a FOH speaker system which visiting technicians are welcome to patch into:

1 x floor standing speaker stack each side of the stage comprising:-

2 x dB Technologies SUB18D Active Sub 1000W

1 x dB Technologies Flexsys F212 PA Speaker 1000W

2 x dB Technologies Flexsys F8 PA Speaker 200W as infills on front of stage

1 x Martin Audio EM76 delay speakers approx 2/3 of the way into the auditorium

Lighting

A lighting rig plan is attached. Visiting technicians are welcome to use the rig free of charge. Lighting patch numbers are shown on the attached lighting rig plan.

Lanterns are spread over 2 fixed FOH front bars; 2 fixed FOH side bars; 2 fixed FOH perches; 3 on-stage LX bars which are lowered using geared hand winches located on the fly floor.

There are 10 x 5amp dips and a variety of 13amp sockets located around the stage.

PLEASE NOTE THAT ALL OUR LIGHTING BAR AND DIP SOCKETS ARE 5AMP ROUND PIN

MAINS ELECTRIC SUPPLY

The main electric supply is situated stage left and comprises:

1 x 63A three phase outlet

3 x 63A single phase outlets

1 x 32A single phase outlet

2 x 16A single phase outlets

MISCELLANEOUS

We have 12 x wooden rostra 1200m x 1200mm x 450mm high; 4 x wooden rostra 1200mm x 1200mm x 225mm high.

44 x red/gold armless chairs

4 x RAT lit music stands

1 x Le Maitre MVS Hazer